Experiment  No.  7

Title:	Implementation of 3 bit synchronous up-down counter using JK flips flops

Batch: B1              Roll No.: 1914078                                                     Experiment  No.: 7

Aim: Implementation of 3 bit synchronous up-down counter using JK flips flops _______________________________________________________________            ___    _

Resources needed: Simulation Software, (Proteus Professional 8).

Theory:

Synchronous Counter Operation

Synchronous counters have a common clock pulse applied simultaneously to all flip-flops.

A 2-Bit Synchronous Binary Counter

Note that both the J and K inputs are connected together. The flip-flop will toggle when both are a 1 (FF0) Also the transition at clock pulse 2 works because of propagation delay effects. Q0 is still high on the input of Q1 at the instant clock 2 hits so FF1 changes state. A short time later clock 2 has propagated through FF0 and it goes low.

Truth table of J K flip flop

| Inputs |  |  | Outputs |  | comments |
| --- | --- | --- | --- | --- | --- |
| J | K | CLK | Q1 | Q’1 |  |
| 0 | 0 | ↑ | Q0 | Q’0 | No change |
| 0 | 1 | ↑ | 0 | 1 | Reset |
| 1 | 0 | ↑ | 1 | 0 | Set |
| 1 | 1 | ↑ | Q’0 | Q0 | Toggle |

Design procedure of Synchronous counter

- Find the number of flip-flops.

- Write the count sequence in tabular form

- Determine the flip-flop inputs which must be present for desired next state from the present state using excitation table of the flip-flops.

- Prepare K-map for each flip-flop input in terms of flip-flop outputs as the input variables.

- Simplify the K-maps and get the minimized expressions for each flip-flop input.

- Connect the circuit using flip-flops and other gates corresponding to the minimized expression

Design A 3-Bit Synchronous up- down Binary Counter using JK flip-flops: (Include the timing diagram)

Solution: (Create a soft copy or use separate assignment sheet and scan for solution)

__________________________________________________________________________

Procedure:

- Design a 3 bit synchronous counter using JK flip-flop as per the design procedure mentioned in theory.

- Simulate the logic design using JK flip-flops on the simulation software.

- Verify the output of the designed and simulated counter for the proper sequence.

- Generate the output timing diagram on simulator.

- Upload Schematic and Timing Diagram generated on Simulator as instructed during lab session.

___________________________________________________________________________

Observations and Results:  Observe the UP-DOWN counter timing diagram as per actual outputs and identify the changes needed in the counter circuit to make the circuit work only as 3bit UP counter.

_____________

_________________________________________________

Outcomes:  Design the combinational and sequential circuits using basic building blocks and MSI devices.

___________________________________________________________________________

Conclusion: We implemented 3 bit synchronous up-down counter using JK flips flops

Grade: AA / AB / BB / BC / CC / CD /DD

Signature of faculty in-charge with date

___________________________________________________________________________

References:

Books/ Journals/ Websites:

- R. P. Jain, “Modern Digital Electronics”, Tata McGraw Hill.

- http://www.electronics-tutorials.ws/counter/count_4.html